I	PURPOSE

The purpose of this procedure is to provide for a system and instructions, and to assign responsibilities for identification of a product inspection status, and release of conforming supplies and services.

II	APPLICATION

This procedure applies to all materials, components, subassemblies, and finished supplies and services.

This procedure concerns Production and Quality Assurance.

III	PROCEDURE

1.	General

1.1	Following every inspection or testing specified, assemblies or components, requiring multiple tests or inspections, are identified to indicate whether they have passed or failed the inspection.  This is to prevent nonconforming product from being used or forwarded for further processing.  The Test and Inspection Plan shall indicate the inspection status on each assembly or component.  All tests and inspection noted, including open and inspect requirements, shall be annotated in the Test and Inspection Plan for each assembly and/or component.

1.2	The inspection status of assemblies is to be identified either by a sticker, tag or marking or by its storage or processing location, or both, to clearly indicate the item is acceptable for continued processing.  When storage or processing location is the only means of identification, the storage or processing location (area, shelf, container, etc.) is presumed to be for conforming materials for continued production.  The storage or processing location (area, shelf, container, etc.) for nonconforming materials shall clearly indicate the item is not acceptable for continued processing by the attaching of a REJECT or NCR tag or sticker.

1.3	Where there are no designated locations or other means for segregating products with different inspection status, or when identification by location may not be sufficiently reliable, products may identified directly by labeling, tagging or marking.

2.	Conforming Products

2.1	Products that have passed the receiving inspection are segregated by being moved to the material stockroom and/or to designated material staging areas in production. Where the storage or staging areas are not sufficiently segregated to prevent intermingling of product with different inspection status, the products are identified with sticker or tag or other identifying marking.

2.3	Being installed in the system under repair segregates products that have passed the final inspection.  In addition, the QA personnel concur by signature at completion of the operational testing phases.

3.	Nonconforming Products

3.1	Products that fail receiving or final inspection are identified with a sticker, tag or marking.  Products failed by in-process inspections may be either identified with a sticker, tag, marking or be just segregated and contained.

3.2	Products failed at receiving are moved to a designated area in the material stockroom or returned immediately to the vendor.  Finished products failed at the final inspection may also be segregated to a designated area.  Products failed by an in-process inspection are either immediately scrapped or are identified in the production area while awaiting disposition, but are removed from the main stream of the production flow.

4.	Authority to Release Products

4.1	The authority to release finished products for shipment is vested with the customer representative who performs the final evaluation.

IV	Associated Documents

SOP–P-03, Product Identification and Traceability

SOP–P-08, Storage Areas

SOP–Q-03, Quality Records