I	PURPOSE

The purpose of this procedure is to provide for a system and instructions, and to assign responsibilities for identification, documentation, and disposition of nonconforming supplies and services.

II	APPLICATION

This procedure applies to all materials, components, subassemblies, and finished supplies and services.

This procedure concerns Quality Assurance and Production.

III	DEFINITIONS

Nonconforming Product: Material, component, subassembly or finished/installed supplies and/or services that do not conform to specified requirements.

IV	PROCEDURE

1	General

1.1	It is a general policy of ManTech that all supplies and service nonconformances are documented, regardless of how insignificant they seem or how easily they can be rectified.  Nonconformance reports are an invaluable tool in tracking performance and trends that give indication where and when cost effective improvement projects may be implemented.

2.	Identification and Documentation

2.1	QA inspection and production personnel are responsible for identifying nonconforming supplies and services in the course of their inspection activities.  In addition, all other personnel, regardless of their other responsibilities, are encouraged to watch for and to identify nonconforming products.

2.2	Only the receiving clerk, QA inspection and supervisory personnel are authorized to initiate discrepancy reports. All other personnel report identified nonconformances to Quality Assurance.

2.3	Whenever a nonconformance is identified, it is documented on a Non-Conformance Report (Form QA-2) and given to the Quality Assurance Manager. The Quality Assurance Manager will issue a serial number and log the necessary information on the Nonconformance Log (Form QA-1). The Quality Assurance Manager will monitor all open Non-Conformance Reports.

  - After the nature of the identified nonconformance is documented, the report number is marked on a hold tag or on the item and the report is further processed with regard to nonconformance review and disposition.

2.5	The nonconformance shall be recorded on the test and inspection plan as a test or inspection result for the work item.

3.	Nonconformance Review and Disposition

3.1	Nonconforming products may be:

- Reworked to meet the specified requirements

- Repaired without meeting specification (may require customer authorization)

- Accepted as-is (may require customer authorization)

- Regraded, with or without repair, for alternative applications

- Scrapped

- Rejected

3.2	The disposition decision may be made on two different levels depending on the nature of the nonconformance and the decision itself:

- When it is obvious that the product must be scrapped or regraded, or when it can be easily reworked without degrading its quality and appearance, QA and production supervisor are authorized to decide what should be done with the nonconforming product.  The decision is documented and authorized in the DISPOSITION block of the nonconformance report.  This action will require the concurrence of the customer’s representative.

- If the repair would in any way affect quality, or if there is a possibility for an accept-as-is decision, the evaluation and disposition are made jointly by Quality Assurance, production supervisors and the customers representative. The disposition decision is documented and authorized in the DISPOSITION block of the nonconformance report.

4.	Control of Repaired and Reworked Product

4.1	Before reworked supplies and services are used in production or are shipped, they are thoroughly inspected to verify that they conform to the same requirements as originally specified.  The manner and scope of inspection are prescribed in the rework instructions.

4.2	Products that are repaired to be accepted with concession, or be regraded for alternative applications, are also inspected to verify that they meet the modified (downgraded) specification.  Regraded products are clearly marked to identify their new status.

5.	Closing Out the Nonconformance Report

5.1	If the disposition decision is to scrap, regrade or accept as-is, the nonconformance report is closed out and filed at that point.

5.2	Rework and repair dispositions require that re-inspection results be entered in the CLOSEOUT box of the nonconformance report.

5.3	Rejected dispositions are closed after product is returned to vendor.

6.	Analysis of Nonconformance Reports and Trends

6.1	Quality Assurance representatives and executive management may periodically review, categorize, quantify, and analyze nonconformance reports to detect trends and identify the possible need for corrective and preventive actions. Standard Operating Procedure, SOP-P-06, Corrective and Preventive Action regulate these activities.

V	ASSOCIATED DOCUMENTS

SOP-P-06, Corrective and Preventive Action