I	PURPOSE

The purpose of this procedure is to provide for a system and instructions and to assign responsibilities for ordering, receiving and inspecting materials as well as releasing conforming supplies and services.

II	APPLICATION

This procedure applies to all materials, components, subassemblies, and finished supplies and services furnished by ManTech where the work authorization, specifications or drawings require such control.

III	PROCEDURE

1.	Material Ordering

1.1	Upon receipt of an approved Delivery Order, the Vice President/Program Manager will review the work scope and authorize preparation of a material requisition for all known materials for the timely completion of the delivery order.

1.2	The Program Manager shall place all material orders and shall provide the purchase order number, vendor and anticipated material delivery date.

2.	Material Receipt and Inspection

  - This system shall provide for inspection at initial receipt and the control of all materials form receipt to issue of materials, equipment and machinery.

  - Receiving personnel shall inspect all items for cracks, nonconforming materials and damages.  At a minimum, the visual inspection shall include:

-	That material composition is as specified on the Purchase Order and vendor packing slip.

- The size is as specified.  The Objective Quality Evidence (OQE) shall specify conformance to Military Specifications as required and numbers of each of the products provided.

-	That quantity, quality, material composition and Military Specifications have been met.  This shall be accomplished by ascertaining that materials ordered on purchase orders were accurately received.  Check the description of each item(s) delivered versus the description on the purchase order.

  - A Material Inspection Record shall be completed upon final inspection and recorded by contract designation.

  - Accepted material shall be identified with a Material Identification Tag or marking with all appropriate information.

  - A Nonconformance Report shall be initiated and the nonconforming material identified by a REJECT/NCR tag or sticker if a nonconformance is noted in the material received.  A copy NCR report shall be forwarded to the Purchasing Agent for disposition.  The REJECTED/NCR material shall be segregated and staged for resolution.  This area shall be labeled to indicate the material within shall not be issued for processing.  If materials are rejected for objective quality evidence has not been received, the condition shall be noted on the nonconformance report and attached to the Purchase Order.  After receipt of the objective quality evidence, the material may be moved or issued for processing.

  - Material received with objective quality evidence shall be accepted if a satisfactory quality history has been established for that particular vendor.

  - If there is doubt the material meets the specifications, it shall be referred to Quality Assurance for resolution. Quality Assurance shall determine the objective quality evidence is valid or will forward samples of the materials for testing.

  - Results and follow-up action taken shall be filed and maintained in the receiving office files.  All records in the receiving office will be maintained for three (3) years for each delivery order. Inspection records will be integrated into the individual job files at the end of each availability and retained for a three (3) year period.  The material samples shall be maintained and made available to the customer.

  - Material having a finite shelf life as prescribe in the applicable Military specification shall be included in receipt inspection and shall be rejected and removed from availability areas when they become outdated.  If outdated materials are received from a vendor, the material shall be rejected and the Purchasing Agent notified.  Reports of the unsatisfactory material shall be maintained in the receiving office files as prescribed above.

  - Documentation of tested material shall indicate that the material or equipment was previously tested and met all requirements and shall contain test data or a statement that such data is on file at the vendor’s plant, by referring to witnessing, inspections present or other verifiable documentation.

  - A complete file of all required and received objective quality evidence shall be maintained in the receiving office file.

3.	Responsibility

3.1	The storeroom supervisor, or the designated representative, shall be responsible for control of all contractor furnished material from receipt until issued to production for utilization on the applicable delivery order.

  - The storeroom supervisor, or the designated representative, shall be responsible for the receipt, inspection and storage of all new contractor furnished material and components or equipment returned after repair by subcontractors.

3.3	The storeroom supervisor or the designated representative shall upon receipt of all contractor-furnished material, components or equipment, inspect the material to ensure compliance with the requirements of the work item.

IV	Associated Documents

SOP–C-03, Purchasing

SOP–P-05, Control of Nonconforming Product

SOP–Q-01, Quality Planning