Specific tissue characteristics and laser parameters used contribute to different tissue effects.  laser radiation parameters can be wavelength, exposure time, applied energy, focal spot size, energy density, and power density (pg 45)

Table 3.9. Wavelengths and photon energies of selected laser systems

Laser type     Wavelength (nm) Photon energy (eV)

ArF 		193 		6.4

KrF 		248 		5.0

Nd:YLF (4ω) 263 		4.7

XeCl 		308 		4.0

XeF 		351 		3.5

Argon ion 	514 		2.4

Nd:YLF (2ω) 526.5 	2.4

He-Ne 	633 		2.0

Diode 	800 		1.6

Nd:YLF 	1053 		1.2

Nd:YAG 	1064 		1.2

Ho:YAG 	2120 		0.6

Er:YAG 	2940 		0.4

CO2 		10600 	0.1

Table 3.11. Sources of UV radiation

Light source Wavelength (nm)

ArF laser 	193

KrF laser	248

Hg lamp 	254

Nd:YLF laser (4ω) 263

Nd:YAG laser (4ω) 266

XeCl laser 	308

XeF laser 	351

The five categories of tissue-laser interaction types are classified today. These are photochem ical interactions, thermal interactions, photoablation, plasma-induced abla tion, and photodisruption. Thermal effects on tissue are vaporization, carbonization, coagulation, Hyperthermia.

Table 3.6. Thermal effects of laser radiation

Temperature Biological effect

37◦C Normal

45◦C Hyperthermia

50◦C Reduction in enzyme activity,

cell immobility

60◦C Denaturation of proteins and collagen, coagulation

80◦C Permeabilization of membranes

100◦C Vaporization, thermal decomposition (ablation)

> 100◦C Carbonization

> 300◦C Melting

3.2.5 Summary of Thermal Interaction:

• Achieving a certain temperature which leads to the desired thermal effect

• Observations can be either coagulation, vaporization, carbonization or melting

• Typical lasers: CO2, Nd:YAG, Er:YAG, Ho:YAG, argon ion and diode lasers

• Typical pulse durations: 1 µs . . . 1 min

• Typical power densities: 10 ... 106 W/cm2

• Special applications: coagulation, vaporization, melting, thermal decomposition, treatment of retinal detachment, laser-induced interstitial thermotherapy

Laser effect examples:

1) Human tooth vaporized with 20 pulses from an Er:YAG laser (pulse

duration: 90 µs, pulse energy: 100 mJ, repetition rate: 1 Hz). (b) Enlargement show ing the edge of ablation

2) Uterine tissue of a wistar rat coagulated with a CW Nd:YAG laser

(power: 10W, bar: 80 µm). Photograph kindly provided by Dr. Kurek (Heidelberg).

3) Human cornea coagulated with 120 pulses from an Er:YAG laser (pulse dura tion: 90 µs, pulse energy: 5 mJ, repetition rate: 1 Hz, bar: 100µm)

4) Tumor metastases on human skin carbonized with a CW CO2 laser

(power: 40W, bar: 1 mm). Photograph kindly provided by Dr. Kurek (Heidelberg).

5) Human tooth carbonized with a CW CO2 laser (power: 1W, bar: 1 mm)

6) Human tooth melted with 100 pulses from a Ho:YAG laser (pulse duration: 3.8 µs, pulse energy: 18 mJ, repetition rate: 1 Hz). (b) Enlargement show ing the edge of the melted zone ig. 3.20.

6) Hole in tooth created by focusing 1000 pulses from a Nd:YLF laser

on the same spot (pulse duration: 30 ps, pulse energy: 1 mJ, repetition rate: 1 kHz).

7) Enlargement showing cubic recrystallization in form of plasma sublimations.

Reproduced from Niemz (1994a).  c 1994 Springer-Verlag

8) Photoablation of corneal tissue achieved with an ArF excimer laser

(pulse duration: 14 ns, energy density: 180 mJ/cm2, bar: 100 µm).

(Citation: Markolf H. Niemz Laser-Tissue Interactions Fundamentals and Applications Third, Enlarged Edition)

-----------------

In response to growing concerns over the production, deployment, and use of laser weapons that cause blinding, the international community in 1995 convened an international conference to restrict the use and transfer of blinding laser weapons. The result was the Fourth Additional Protocol to the 1980 United Nations Convention on Certain Conventional Weapons. In doing so, a significant step was taken towards restricting the intentional blinding of combatants in time of war.

(The Legal Status of Laser Weapons That Blind, Jeffrey S. Morton, Volume 35, Issue 6)